An Integrated Multi-Modal Artificial Intelligence and Internet of
Things Framework for Intelligent Dry Fish Processing

| 1{st Dept. of Information Technology Sri Lanka Institute of Information Technology Malabe, Sri Lanka email address or ORCID | 2{nd Dept. of Information Technology Sri Lanka Institute of Information Technology Malabe, Sri Lanka email address or ORCID | 3{rd Dept. of Information Technology Sri Lanka Institute of Information Technology Malabe, Sri Lanka email address or ORCID | 4{th Dept. of Information Technology Sri Lanka Institute of Information Technology Malabe, Sri Lanka email address or ORCID |
| --- | --- | --- | --- |

Abstract—Traditional dry fish processing depends heavily on operator experience, manual inspection, and variable environmental conditions, resulting in inconsistent product quality, spoilage risk, processing inefficiencies, undocumented waste, and incomplete batch histories. Existing intelligent food processing approaches commonly address environmental monitoring, drying prediction, visual quality assessment, automated control, and digital traceability as independent functions. This paper presents an integrated Artificial Intelligence and Internet of Things framework that unifies these capabilities within a single batch centric dry fish processing lifecycle. The proposed system combines real time temperature, relative humidity, gas, and continuous weight sensing with machine learning based drying parameter prediction, dynamic remaining time estimation, spoilage risk assessment, computer vision based quality grading and defect detection, automated drying control, salt and waste management, and digital traceability. A unique batch identifier links preparation information, sensor observations, predictions, visual assessments, control actions, and sustainability records throughout processing. Environmental, physical, chemical, and visual information are collectively used to represent the evolving state of a drying batch, while safety constraints remain independent of model generated decisions. The resulting framework establishes a sense, predict, assess, decide, control, verify, and trace processing cycle designed to transform experience dependent dry fish production into a measurable, intelligent, and traceable process. The paper reports the implementation status of each subsystem and defines an evaluation protocol spanning predictive model performance, visual classification performance, environmental control stability, drying completion accuracy, system reliability, and traceability completeness.

Index Terms—artificial intelligence, computer vision, dry fish processing, Internet of Things, machine learning, multi-modal sensing, predictive analytics, smart drying, traceability

I.  Introduction

Drying is among the oldest methods of food preservation. By reducing available moisture it suppresses microbial growth and enzymatic activity, extending shelf life and easing storage and transport. In Sri Lanka, dried fish is an important processed seafood product contributing to household food consumption and to the livelihoods of coastal processing communities.

Despite this importance, conventional processing remains highly dependent on manual practice. Operators determine preparation requirements, salting duration, drying conditions, and final quality through experience and visual observation, while open processing environments expose drying to changing temperature and humidity. The consequences are interlinked: environmental fluctuation changes the rate of moisture removal; inappropriate conditions cause under drying, over drying, or deterioration; manual grading varies between operators; and waste and batch histories are rarely documented.

Recent advances in the Internet of Things (IoT), artificial intelligence (AI), machine learning (ML), and computer vision offer routes to replace experience dependent judgement with measurable, data driven workflows. Implementing these technologies independently, however, limits their combined value. A drying time prediction is more useful when continuously updated from live observations. A visual quality assessment is more informative when interpreted alongside the environmental history and physical progression of the same batch. A control decision is more defensible when the evidence behind it is retained in the batch record.

This research therefore proposes a unified intelligent dry fish processing framework that treats sensing, prediction, visual assessment, control, sustainability, and traceability as stages of the same processing lifecycle. The system observes the drying process through multiple data modalities,

X(t) = [T(t), H(t), W(t), G(t), I(t), B],	(1)

where T(t) represents chamber temperature, H(t) represents relative humidity, W(t) represents batch weight, G(t) represents a gas or odour measurement, I(t) represents an image observation of the fish, and B represents batch specific information. These observations support a set of intelligent outputs,

Y(t) = [R̂(t), P̂s(t), Q(t), C(t), D(t)],	(2)

where R̂(t) represents predicted remaining drying time, P̂s(t) represents spoilage risk, Q(t) represents the derived visual quality and defect state, C(t) represents control decisions, and D(t) represents drying completion state.

The principal contribution of this research is a unified batch centric, multi-modal intelligent dry fish processing architecture that combines environmental sensing, physical weight progression, gas based spoilage indicators, predictive machine learning, computer vision based visual quality assessment, automated drying control, sustainability management, and event level traceability within a single processing lifecycle. Unlike isolated monitoring or prediction approaches, the proposed framework enables heterogeneous observations and model generated outputs to contribute to a shared representation of the current batch state, and preserves the resulting processing decisions within a complete digital history.

The primary research question is whether the integration of heterogeneous sensing, predictive analytics, visual quality assessment, and automated control can provide a practical intelligent processing framework for improving dry fish processing while maintaining a complete digital batch history.

II.  Related Work

A.  IoT and Intelligent Food Drying

IoT technologies have increasingly been introduced into food drying to support real time monitoring and environmental control. Mishra et al. [1] developed a drying system using the Internet of Things for food quality monitoring and controlling, and demonstrated that real time monitoring and control can maintain environmental conditions and generate warnings when abnormal processing conditions occur. Closer to the present application, Alvinika et al. [2] designed automatic fish drying equipment in which fuzzy logic regulates the drying chamber from temperature and humidity observations, confirming that closed loop control of a fish dryer is achievable with low cost hardware.

Such systems represent an important transition from conventional drying toward instrumented processing. Nevertheless, environmental measurements primarily describe the drying environment rather than the complete physical and visual state of the product. A recent review of intelligent monitoring, predicting, and control technology for food drying [3] similarly observes that many deployed systems remain reactive, and identifies closed loop integration of sensing and prediction as an open direction.

B.  Machine Learning for Drying Prediction

Drying is a dynamic process influenced by temperature, relative humidity, product characteristics, and elapsed processing time. Rather than relying on a predetermined duration, a predictive system updates estimates as new observations become available. For a sequence of observations

X1:t = {X1, X2, …, Xt},	(3)

a regression model estimates remaining processing time as

R̂t = f(X1:t).	(4)

This approach has been validated in adjacent domains. Rakholia et al. [4] predicted meat drying time from historical process data using an ensemble machine learning model for production planning, and Ahmad et al. [5] estimated grain moisture in a high temperature paddy drying silo from environmental parameters, reporting that a shallow neural network gave the best accuracy for real time use. Classical thin layer drying models remain the physical baseline against which such data driven estimators are judged; Ghanem et al. [6] fitted thin layer models to solar dried salted tilapia fillets and reported close agreement with measured moisture ratios. The present work adopts gradient boosted regression ensembles [7], which suit the modest tabular datasets realistic for small scale processing trials.

A common limitation across this literature is that predictions are generated offline or as advisory outputs. Comparatively few systems close the loop by returning predicted parameters to the drying equipment as control setpoints.

C.  Sensing Product State: Odour and Vision

Beyond the drying environment, the state of the product itself can be observed through chemical and visual signals. Volatile compounds released during protein degradation provide an objective spoilage indicator; Setyagraha et al. [8] constructed a portable electronic nose using low cost metal oxide gas sensors and machine learning classifiers to discriminate seafood freshness levels, supporting the use of an inexpensive gas sensor as an odour proxy.

Computer vision is a complementary non destructive technology for automated food inspection, since images contain information about colour, surface appearance, shape, shrinkage, and defects. Chakravartula et al. [9] combined computer vision based smart monitoring with control for food drying in a study on carrot slices, demonstrating the potential of combining visual and physical observation. In the seafood domain, Karmaker et al. [10] applied convolutional neural networks with transfer learning to fish freshness classification.

A vision model alone, however, does not provide information about the environmental history that produced the observed appearance, which motivates interpreting visual evidence alongside process measurements.

D.  Intelligent Process Control

Control of drying equipment has progressed from fixed timers through threshold rules to fuzzy and model based strategies [2], [3]. A recurring design principle in this literature is that equipment protection should not depend on the correctness of a predictive model. The present work adopts that principle explicitly by separating the safety envelope from the prediction path.

E.  Digital Traceability and Sustainability

Cromwell et al. [11] systematically reviewed digital technologies for traceability and transparency in global fish supply chains, covering IoT, radio frequency identification, and blockchain approaches, and highlighted integration as a prerequisite for higher levels of traceability. Tsang et al. [12] demonstrated a concrete architecture combining blockchain and IoT for perishable food traceability with shelf life management.

Much of that research focuses on catch, transportation, cold chain management, and distribution, and treats the processing step as a single opaque event.

F.  Research Gap

Existing studies demonstrate that IoT monitoring [1], [2], intelligent drying control [3], machine learning prediction [4], [5], gas based spoilage assessment [8], computer vision [9], [10], and digital traceability [11], [12] individually contribute to improved food and fish processing systems. The research opportunity addressed here is therefore not the independent application of these technologies.

Three specific gaps motivate the present design. First, predictive models are commonly evaluated offline rather than used to set the operating parameters of the drying equipment itself. Second, drying temperature is usually treated as a fixed operator chosen input, whereas it is more naturally a quantity to be predicted from batch characteristics. Third, traceability research records supply chain custody rather than the intelligent decisions taken during processing.

III.  Proposed Integrated Framework

A.  Architecture and Batch Centric Data Model

The proposed system is organised around the lifecycle of an individual dry fish batch rather than around separate applications. A drying chamber provides environmental, chemical, and physical observations; analytical services convert those observations into predictions; a control layer converts predictions into actuator decisions subject to safety constraints; a vision subsystem contributes visual evidence about product state; and a traceability layer preserves the resulting record.

Each batch receives a unique identifier BIDi, which remains associated with all information generated during processing, giving a record

Bi = {Pi, Ei, Wi, Ai, Vi, Si, Ti},	(5)

where Pi denotes preparation information, Ei environmental observations, Wi weight progression, Ai analytical predictions, Vi visual assessments, Si sustainability information, and Ti processing and traceability events.

B.  End to End Processing Workflow

The architecture follows a sense, predict, assess, decide, control, verify, and trace cycle. Sensing supplies environmental, chemical, and physical observations. Prediction estimates drying parameters, remaining time, and spoilage risk. Assessment contributes visual evidence of product state. Decision and control convert the combined state into actuator commands. Verification evaluates completion from multiple sources of evidence. Tracing preserves every significant event against the batch identifier.

IV.  Multi-Modal Data Acquisition

A.  Environmental Sensing

Temperature and relative humidity are sampled continuously. Environmental state at time t is

Et = [Tt, Ht].	(6)

A safe operating condition is represented by

CE(t) =
1 if Tmin ≤ Tt ≤ Tmax ∧ Ht ≤ Hmax; 0 if otherwise.	(7)

Persistence rules are applied before generating environmental alerts, so isolated sensor noise does not immediately trigger an intervention.

B.  Weight Progression

A load cell continuously measures batch weight. The normalised weight ratio, percentage reduction, and rate of change are

RW(t) = (Wt)/(W0), 
LW(t) = (W0 - Wt)/(W0) × 100, 
rW(t) = (Wt-Δt - Wt)/(Δt),	(8)

where W0 is the initial batch weight. Weight progression describes physical evolution that cannot be obtained from chamber temperature alone.

C.  Gas and Odour Sensing

A metal oxide gas sensor provides an odour related signal associated with volatile compounds released during protein degradation. The raw reading is mapped to discrete smell levels using configurable prototype thresholds, which require experimental calibration against reference freshness measurements before they can be treated as established limits.

D.  Image Acquisition and Processing

Images of the batch are captured for visual assessment. Each image is resized to a fixed input resolution and normalised before inference. The associated batch identifier and timestamp are retained so that a visual result can be interpreted alongside the process measurements recorded at the same point in the lifecycle.

V.  AI Based Process Intelligence

A.  Drying Parameter Prediction

Before drying begins, the system predicts the recommended chamber temperature and the expected total drying time from conditions known in advance,

[θ̂, R̂0] = fφ(variety, W0, H0,
G0).	(9)

This differs from approaches that treat drying temperature as a fixed operator chosen input: here temperature is itself a predicted quantity, which in turn determines the expected duration. The predictor is implemented as a multi output gradient boosted regression ensemble [7]. Where live sensor readings are unavailable at prediction time, documented default values are substituted and the result is marked accordingly, so an operator can distinguish a sensor grounded estimate from a fallback estimate.

B.  Dynamic Remaining Time Prediction

During drying, accumulated observations are used to re estimate progression. At time t the predictive feature vector includes

Zt = [Tt, Ht, Wt, RW(t), rW(t), τt, B],	(10)

where τt is elapsed drying time, and the estimate is

R̂t = fθ(Z1:t).	(11)

The prediction is updated periodically rather than computed once at the start of processing.

C.  Spoilage Risk Prediction

The same observation history supports a risk model,

P̂risk(t) = gψ(Z1:t),	(12)

whose output is mapped to low, medium, and high categories. Rather than presenting only a risk label, the interface reports the contributing factors, giving each reading with its value, expected range, and direction of influence, so that model limitations remain visible to the operator.

D.  Visual Quality and Defect Assessment

Environmental and weight measurements describe the drying environment and physical progression but do not directly characterise visible changes in product quality. The framework therefore incorporates a computer vision subsystem for automated visual assessment during and after processing.

Images captured from the active batch are processed by a trained vision model,

Qt = Fω(It),	(13)

where It is the image captured at time t, Fω is the trained model, and Qt is the predicted visual quality state.

The assessment supports quality grading and identification of observable undesirable conditions such as abnormal discoloration, uneven drying, mould like appearance, and visible damage, according to the classes represented in the training dataset. A vision result is represented as

Vt = {BID, t, qt, dt, ct},	(14)

where qt is the predicted quality class, dt is the detected visual condition or defect, and ct is the prediction confidence.

The subsystem is implemented as a segmentation model, so each detection carries a region mask in addition to a class and confidence, which localises a defect within the batch rather than only reporting its presence. Unlike an isolated image classification application, the vision result is designed to become part of the same processing state used by the wider system, enabling visual evidence to be interpreted together with temperature, humidity, gas measurements, weight progression, drying time predictions, and spoilage risk.

E.  Waste and Salt Intelligence

At batch creation the platform estimates expected cleaning waste and the required salt quantity and salting duration, storing both against the batch rather than as isolated calculations. Salting is monitored to completion, and drying cannot begin until the salting stage is recorded complete, which enforces process order within the platform rather than relying on operator memory.

VI.  Intelligent Decision and Drying Control

A.  Multi-Modal State Estimation

Environmental, physical, chemical, predictive, and visual information are combined into an overall processing state,

St = G(Et, RW(t), rW(t), Gt, R̂t,
P̂risk(t), Qt).	(15)

The system therefore characterises the batch through four complementary evidence sources: environmental conditions, physical weight progression, a chemical odour proxy, and visual appearance. Conceptual processing states include initialisation, active drying, environmental risk condition, quality risk condition, approaching completion, completion verification, and drying complete.

B.  Actuator Control

A control action is generated as

Ut = π(St, Et),	(16)

where Ut represents heater activation, heater reduction, ventilation activation, continuation, protective intervention, or process termination.

C.  Safety Constraints

Two properties are enforced by construction. First, every recommended temperature is clamped to a configured operating envelope before reaching the controller, so an erroneous prediction cannot command an unsafe setpoint. Second, the chamber validates sensor completeness independently and transitions to a fault state that de energises the heater whenever readings are missing or implausible. Safety constraints therefore remain independent of model output, consistent with the principle that hardware protection should not depend on the software control layer.

D.  Completion Detection

Completion is treated as a multiple evidence decision,

Dt = FD(RW(t), rW(t), R̂t, Et, Qt).	(17)

Drying completion is not determined by a single fixed timer or an isolated sensor threshold. Weight progression identifies physical advancement toward completion, remaining time prediction provides predictive evidence, environmental measurements describe current chamber conditions, and visual assessment provides complementary evidence about the observable product state. Agreement among these modalities increases confidence in the completion decision, while conflicting evidence can trigger continued monitoring or operator verification.

The weight loss threshold used by the prototype is a configurable parameter rather than a scientifically established constant, and requires per variety experimental validation.

VII.  Traceability and Sustainability

Processing waste is maintained as structured batch information. A waste event contains

We = {type, quantity, time,
method, BID},	(18)

and each significant processing event is stored as Tk = (BID, tk, typek, valuek, sourcek). The resulting timeline spans batch creation, preparation, salt record, drying start, sensor observations, predictions, risk alerts, control actions, visual assessments, completion, and waste record. The objective is not merely to identify where a product originated, but to retain a digital history of how the batch was processed.

VIII.  System Implementation

A.  Hardware

The prototype uses a microcontroller as the primary edge device, streaming a periodic sensor block over a serial link. Temperature and relative humidity sensing uses a digital environmental sensor, supplemented by a separate temperature probe. A metal oxide gas sensor provides the odour proxy, and a load cell with a signal conditioning amplifier provides continuous weight measurement. Relay circuitry interfaces the controller with heating and ventilation mechanisms.

B.  Backend Services and Data Flow

The platform follows a service oriented architecture, with separate services for authentication, batch and waste management, environmental monitoring and actuator control, and predictive analytics, communicating over HTTP interfaces and persisting to a document database.

Starting a drying run is a two phase operation. The predicted temperature, humidity target, and duration are first registered with the chamber as a control profile. The session starts only if the chamber validates its own sensor state, and only after the chamber confirms it is running does the analytics service record the batch as drying.

C.  Vision Subsystem

The vision subsystem is implemented as a segmentation model served by a dedicated inference service exposing health, model metadata, and prediction endpoints, together with a web application for image collection, annotation, dataset export, and model version deployment. The deployed network is a medium capacity single stage segmentation architecture trained at 640×640 resolution with geometric and photometric augmentation appropriate to small datasets. Inference returns, for each detection, a class label, a confidence score, a bounding box, and a polygon mask.

The subsystem is presently operational as a standalone service. Its integration into the batch lifecycle, so that Qt contributes directly to state estimation and completion decisions as specified in (17) and (19), is ongoing work. The architecture and interfaces required for that integration are defined above.

D.  User Application

A cross platform mobile application presents a unified representation of the active batch, displaying batch identification, live readings, drying progression, predicted remaining time, spoilage risk, actuator status, alerts, and traceability history, so the operator does not interact separately with individual subsystems.

E.  Reconciling Logical and Physical State

Prototype operation exposed a failure mode worth reporting. The analytics service and the chamber maintain separate notions of whether drying is occurring. When the chamber halted, through a safety fault or a lost sensor link, the analytics service continued to report an active batch and the interface continued a countdown derived from the pre drying estimate. The displayed state was internally consistent yet physically false. The resolution was to make the chamber authoritative, so that the client suspends the countdown and explains the discrepancy whenever the two disagree. A service holding a logical record of an activity must not be permitted to assert that the activity is physically occurring.

IX.  Experimental Methodology

A.  Objectives and Data Collection

Evaluation establishes whether individual intelligent functions perform accurately, whether they exchange information reliably, and whether integrated operation provides measurable benefit. For every batch the following are recorded: fish variety, initial weight, preparation information, temperature, humidity and weight time series, image observations, actual drying duration, model predictions, actuator changes, and waste and salt information.

Training, validation, and test partitions must be separated at the batch level. Measurements within a single physical trial are strongly autocorrelated, so partitioning at the sample level would allow observations from one trial to appear in both training and test sets, inflating apparent accuracy.

B.  Predictive Evaluation

Remaining time prediction is evaluated using mean absolute error and root mean squared error,

MAE = 1/NΣ|yi - ŷi|, 
RMSE = √{1/NΣ(yi - ŷi)^2}.	(19)

Spoilage risk classification is evaluated using precision, recall, F1 score, and confusion matrices.

C.  Computer Vision Evaluation

The visual quality assessment model is evaluated on an independent test set using accuracy, precision, recall, F1 score, and a confusion matrix,

P = TP/(TP+FP), R = TP/(TP+FN), 
F1 = 2PR/(P + R).	(20)

For the segmentation formulation, mean average precision at intersection over union thresholds of 0.5 and 0.5 to 0.95 is additionally reported.

To assess practical robustness, testing includes variation in illumination, camera position, fish orientation, and surface appearance where represented by the collected dataset. Particular attention is given to class wise performance, because high overall accuracy alone may conceal poor detection of infrequently occurring defect classes. Evaluation requires a held out test set that is disjoint from both the training and validation partitions.

D.  Control and Traceability Evaluation

Environmental control is evaluated using mean absolute temperature deviation,

MAET = 1/NΣ|Ti - Tset,i|,	(21)

together with time outside the permitted humidity range, actuator response latency, sensor availability, and communication success rate. Traceability completeness is

TC = (Nrecorded)/(Nexpected) × 100.	(22)

E.  End to End Evaluation

The integrated system is compared with a baseline such as conventional operator managed processing or threshold based control, investigating drying duration, unsafe humidity exposure, temperature stability, completion detection error, manual interventions, risk event detection, batch record completeness, and end to end reliability.

X.  Results and Discussion

Measured experimental results are not reported at this stage. The drying predictors are presently calibrated against a limited number of validated physical runs, and the vision model has been trained but not yet evaluated on a held out test set that is disjoint from its training and validation data. Reporting performance figures under these conditions would misrepresent their empirical standing. Table~I and Table~II define the intended reporting structure.

TABLE I
Intelligent Model Performance

| Function | Primary metric | Result |
| --- | --- | --- |
| Pre drying temperature | MAE (° C) | Pending |
| Pre drying duration | MAE (min) | Pending |
| Remaining time prediction | MAE, RMSE | Pending |
| Spoilage risk prediction | F1 score | Pending |
| Visual quality grading | Accuracy, F1 | Pending |
| Defect detection | Precision, recall, F1 | Pending |

TABLE II
Baseline and Proposed System Comparison

| Metric | Baseline | Proposed |
| --- | --- | --- |
| Temperature MAE | Pending | Pending |
| Unsafe humidity exposure | Pending | Pending |
| Completion error | Pending | Pending |
| Drying duration | Pending | Pending |
| Manual interventions | Pending | Pending |
| Traceability completeness | Pending | Pending |

The evaluation should not claim that the integrated system is superior simply because it contains more technologies. It should determine whether multi-modal information provides practical value unavailable from single mode processing. Two cases illustrate the argument. A humidity increase alone identifies an environmental event; the same increase coinciding with a reduced weight loss rate and rising predicted spoilage risk is materially stronger evidence for intervention. Likewise, a weight threshold alone suggests possible completion, whereas agreement between weight stabilisation, the remaining time prediction, and the visual quality state supports termination far more strongly. Disagreement among modalities is equally informative and should be analysed rather than suppressed.

Prototype experience further indicates that integration benefits are contingent on sensing reliability. Intermittent load cell dropouts repeatedly triggered protective faults that terminated otherwise healthy runs. No application layer logic compensates for an unreliable measurement chain, so sensor robustness is a precondition for intelligent control rather than an implementation detail.

XI.  Limitations

Several limitations should be considered. First, model performance depends strongly on the number and diversity of real drying trials. The present predictors are anchored to a small set of validated runs, with variety specific profiles partly parameterised rather than measured, so results should not be generalised to all dry fish products.

Second, weight reduction is a useful indicator of drying progression but should not be interpreted as equivalent to laboratory measured moisture content without experimental validation.

Third, low cost sensors introduce measurement uncertainty and require calibration. An uncalibrated load cell directly corrupts both completion detection and remaining time estimation, because both depend on the same weight signal.

Fourth, the vision model has been trained but not yet evaluated on an independent held out test set, and its dataset scale has not been established. Quantitative visual performance claims are therefore deferred.

Fifth, several operating thresholds in the prototype, including the weight loss completion criterion, the gas to smell level mapping, and the temperature envelope, are configurable engineering parameters rather than experimentally established constants.

Finally, the integrated architecture increases system complexity. Failure of communication, of individual sensors, or of analytical services must be considered when evaluating practical deployment.

XII.  Conclusion and Future Work

This paper presented an integrated AI and IoT framework for intelligent dry fish processing that combines environmental sensing, continuous weight monitoring, gas based spoilage indicators, machine learning based drying prediction, computer vision quality assessment, automated drying control, sustainability management, and digital traceability within a unified batch lifecycle.

The proposed architecture observes both the processing environment and the changing state of the product. Temperature and humidity describe environmental conditions, continuous weight measurements characterise physical drying progression, gas sensing provides an additional spoilage related signal, predictive models estimate drying requirements and process risk, and computer vision provides complementary evidence about visible product quality. These heterogeneous observations contribute to a common processing state from which control, completion, quality, and traceability decisions can be generated.

Beyond the architecture itself, two findings are practically significant. Predicted parameters must be constrained by a safety layer that operates independently of model output. In addition, the component holding a logical record of an activity must never be treated as authoritative about whether that activity is physically occurring, because the device itself is the only reliable authority.

The resulting sense, predict, assess, decide, control, verify, and trace architecture demonstrates how previously isolated intelligent functions can be organised into a single cyber physical processing framework. Future work should complete the integration of the vision subsystem into the batch lifecycle, expand the number of experimentally validated drying batches, investigate multi-modal fusion techniques, establish laboratory reference measurements for moisture and product quality, improve sensor robustness, and validate the framework across multiple fish varieties in real small scale processing environments.

Acknowledgment

The authors would like to acknowledge the academic supervisors, domain experts, dry fish processors, and members of the Smart Karawala research project who support the design, implementation, and evaluation of the proposed system.

References

[1]	N. Mishra, S. K. Jain, N. Agrawal, N. K. Jain, N. Wadhawan, and N. L. Panwar, “Development of drying system by using internet of things for food quality monitoring and controlling,” Energy Nexus, vol. 11, art. no. 100219, Sep. 2023.

[2]	Y. Alvinika, D. B. Setyohadi, and M. Sulistyoningsih, “IoT-based monitoring and design of automatic fish drying equipment using fuzzy logic,” IOP Conf. Ser.: Earth Environ. Sci., vol. 704, no. 1, art. no. 012042, 2021.

[3]	J. Guo, X. Zhang, Y. Liu, J. Wu, H. Xu, H. Xiao, Z. Ai, and P. Gong, “Intelligent monitoring, predicting, and control technology of food drying: recent advances, challenges, and future prospects,” Food Control, vol. 180, art. no. 111666, 2026.

[4]	R. Rakholia, A. L. Suárez-Cetrulo, M. Singh, and R. S. Carbajo, “AI-driven meat food drying time prediction for resource optimization and production planning in smart manufacturing,” IEEE Access, vol. 13, pp. 22420--22428, 2025.

[5]	F. Ahmad, M. S. Younis, R. U. Zahid, and L. A. Shahid, “Machine learning based grain moisture estimation for real-time monitoring of high-temperature paddy drying silo,” in Proc. IEEE 23rd Int. Multitopic Conf. (INMIC), 2020, pp. 1--6.

[6]	T. H. M. Ghanem et al., “Thin-layer modeling, drying parameters, and techno-enviro-economic analysis of a solar dried salted tilapia fish fillets,” Sci. Rep., vol. 15, art. no. 5073, 2025.

[7]	J. H. Friedman, “Greedy function approximation: a gradient boosting machine,” Ann. Statist., vol. 29, no. 5, pp. 1189--1232, 2001.

[8]	M. R. M. Setyagraha, H. A. Nurqamaradillah, L. M. Hermawan, N. R. Pratama, L. Novamizanti, and D. R. Wijaya, “A portable real-time electronic nose for evaluating seafood freshness using machine learning,” IEEE Access, vol. 13, pp. 99583--99602, 2025.

[9]	S. S. N. Chakravartula, A. Bandiera, M. Nardella, G. Bedini, P. Ibba, R. Massantini, and R. Moscetti, “Computer vision-based smart monitoring and control system for food drying: a study on carrot slices,” Comput. Electron. Agricult., vol. 206, art. no. 107654, Mar. 2023.

[10]	R. Karmaker, M. A. Tinky, and S. Akter, “Deep learning and image processing approach for fish freshness detection,” in Proc. IEEE Int. Conf. Power, Elect., Electron. Ind. Appl. (PEEIACON), 2024, pp. 1--6.

[11]	J. Cromwell, C. Turkson, M. Dora, and F. A. Yamoah, “Digital technologies for traceability and transparency in the global fish supply chains: a systematic review and future directions,” Marine Policy, vol. 178, art. no. 106700, Aug. 2025.

[12]	Y. P. Tsang, K. L. Choy, C. H. Wu, G. T. S. Ho, and H. Y. Lam, “Blockchain-driven IoT for food traceability with an integrated consensus mechanism,” IEEE Access, vol. 7, pp. 129000--129017, 2019.